SAFEBOX STAFF POLO

FINAL MANUFACTURING & PRINT SPECIFICATION

| APPROVED STYLE Design 2 - white professional polo | FRONT SAFEBOX + small worker/storage icon | BACK Complete official logo, undistorted |
| --- | --- | --- |

Official website: https://www.gosafebox.com

# 1. GARMENT CONSTRUCTION

| ITEM | REQUIRED SPECIFICATION |
| --- | --- |
| Garment | Premium short-sleeve staff polo; professional corporate cut, not a sports jersey. |
| Fabric | Breathable moisture-wicking polyester pique, 180-220 gsm, wrinkle-resistant and colourfast. |
| Main body | White. |
| Side panels | SAFEBOX green, running from underarm to lower hem on both sides. |
| Piping | Thin black piping separating the green side panels from the white body. |
| Collar / placket | White collar; green inside collar and green inner placket accent; three white buttons. |
| Sleeve cuffs | White cuff with one green stripe and one black stripe. |
| Fit | Modern professional fit. Supplier must provide male and female size samples or size chart before order. |
| Finish | Straight stitching, no loose threads, no puckering around embroidery or transfers. |

# 2. LOGO PLACEMENT

Front - left chest: Use the SAFEBOX wordmark with the small worker/storage-unit icon positioned above it as shown in the approved mockup. Finished artwork area approximately 90-105 mm wide. Keep it level and centred on the left chest. No website and no “SELF STORAGE” line on the front.

Back - centred: Use the complete official SAFEBOX logo: storage unit, worker, SAFEBOX, SELF STORAGE and GoSafeBox.com. Target width 240-260 mm, centred horizontally and positioned below the collar. Preserve the exact aspect ratio. Do not stretch, compress, redraw, retype or alter any element.

Sleeves: No extra sleeve logo is required in the final design unless approved in writing after the first physical sample.

# 3. DECORATION METHOD

Front: High-quality embroidery preferred. Because the small icon contains fine detail, the supplier must submit a stitch simulation. If detail is not clean, use premium heat-transfer/DTF for the complete front mark instead of simplifying the logo.

Back: Premium DTF, screen print or equivalent full-colour transfer suitable for repeated commercial washing. Minimum source resolution 300 dpi at final print size. A vector redraw may be prepared only by tracing the supplied official artwork without changing proportions or colours.

# 4. QUALITY & APPROVAL

Produce one complete size-L physical sample before bulk production.

Send clear front, back and close-up photographs plus actual measurements of both logos.

Logo placement tolerance: +/- 3 mm. Colour must remain consistent across the production run.

No bulk production until written approval of the physical sample.

Print/transfer must not crack, peel or fade after normal washing; supplier to state its wash guarantee.

# 5. OFFICIAL ARTWORK & COLOURS

The actual reference artwork supplied by SAFEBOX is included in the accompanying Artwork folder. These files are the visual source of truth. Do not use AI-generated mockup logos for production.

| COMPLETE BACK LOGO | WORKER ICON |
| --- | --- |

| COLOUR | HEX | RGB | PRODUCTION NOTE |
| --- | --- | --- | --- |
| SAFEBOX Green | #61993B | 97 / 153 / 59 | Primary brand reference. Supplier must colour-match from a physical or printed proof before production. |
| Black | #000000 | 0 / 0 / 0 | Wordmark, outlines and piping. |
| White | #FFFFFF | 255 / 255 / 255 | Main garment body and negative-space logo details. |
| Box Gold | Use supplied artwork | Use supplied artwork | Do not recolour the box. Match directly from the official logo artwork. |

# 6. FILES INCLUDED

SAFEBOX_Full_Logo_Official_Reference.png, SAFEBOX_Worker_Icon_Official_Reference.png and SAFEBOX_Polo_Approved_Mockup.png.

IMPORTANT FOR THE PRINT SHOP: These are high-resolution raster references. Before mass production, submit a final vector production proof (AI, EPS, SVG or vector PDF) for SAFEBOX approval. Do not substitute fonts, distort the logo or modify the artwork.